ARTIC Multiplex PCR Protocol

Scheme: artic-inrb-mpox/400/v1.0.0

Due to the large number of primers included in each pool, the 100 µM primer pool stocks do not need to be diluted as this would result in too large a volume for the multiplex PCR reaction.

Therefore, the 100 µM primer pool stocks are added directly into the Multiplex PCR reaction mix.

Multiplex PCR:

- Set up two PCR reactions per sample as follows in strip-tubes or plates:

  - Depending on available reagents use Reaction Mix A for M0493 (Q5® Hot Start High-Fidelity DNA Polymerase) or Reaction Mix B for M0494 (Q5® Hot Start High-Fidelity 2X Master Mix).

Reaction Mix A:

| Component | Reaction 1 | Reaction 2 |
| --- | --- | --- |
| 5X Q5 Reaction Buffer | 5 µl | 5 µl |
| 10 mM dNTPs | 0.5 µl | 0.5 µl |
| Q5 Hot Start DNA Polymerase | 0.25 µl | 0.25 µl |
| Pool 1 (100 µM) | 2.2 µl | 0 µl |
| Pool 2 (100 µM) | 0 µl | 2.2 µl |
| Nuclease-free Water | 14.55 µl | 14.55 µl |
| Total | 22.5 µl | 22.5 µl |

OR

Reaction Mix B:

| Component | Reaction 1 | Reaction 2 |
| --- | --- | --- |
| Q5 Hot Start High-Fidelity 2X Master Mix | 12.5 µl | 12.5 µl |
| Pool 1 (10 µM) | 2.2 µl | 0 µl |
| Pool 2 (10 µM) | 0 µl | 2.2 µl |
| Nuclease-free Water | 7.8 µl | 7.8 µl |
| Total | 22.5 µl | 22.5 µl |

- Gently mix by pipetting and pulse spin the tube to collect liquid at the bottom of the tube.

- Add 2.5 µl cDNA (or DNA) to each of the PCR reactions, gently mix by pipetting and pulse spin the tube to collect liquid at the bottom of the tube.

- Set up the following program on the thermal cycler:

  - Cycle number should be 25 for Ct 18 -21 up to a maximum of 35 cycles for Ct 35.

| Step | Temperature | Time | Cycles |
| --- | --- | --- | --- |
| Heat Activation | 98 °C | 30 Seconds | 1 |
| Denaturation | 98 °C | 15 Seconds | 25 – 35 |
| Annealing | 65 °C | 5 minutes | 25 – 35 |
| Hold | 4 °C | ∞ | 1 |

The pools can now be combined and sequenced.

For ONT sequencing, we recommend following: One-pot native barcoding of amplicons v4 (LoCost) V.2

https://dx.doi.org/10.17504/protocols.io.kxygxebydv8j/v2